File Transfer between the Tandy 2000 and an

IBM PC-AT or compatible with 1.2M drives

running PC-DOS 3.2 or later.

by

David Ogle

[75676,2612]

June 14, 1987

It is possible, with the aid of the INTRCPT utility included with

this archive (from IBMHW forum on CompuServe), to read & write Tandy

2000 720K disks in an AT or compatible with a 1.2M drive.  You may

even use the very hardware dependent DISKCOPY command under PC-DOS

with the aid of INTRCPT (see the INTRCPT documentation for more

details).  INTRCPT requires DOS 3.2 or greater to work, and the Tandy

2000 720K disks must be zapped to allow the transfer.  The Media

descriptor in the boot record and the FAT ID on the first FAT must be

zapped to F9h in order for the disks to be recognized by PC-DOS as

80-track diskettes.  A utility is included in this archive to allow

this zapping to be done in a reliable manner (ZAPMID).  The following

command string will do the zapping for a disk in drive A:

ZAPMID A:

The ZAPMID utility may be executed on the 2000 or on the AT -- it doesn't

matter which.  To zap the disk back afterwards, use the command:

ZAPMID A: /t

However, the reverse zap really does not appear to be necessary.  I

have found that the zapped disks work just fine under MS-DOS 2.11.03

and probably will on prior versions of DOS as well.  The Turbo C

source for ZAPMID is included in this archive -- to get more

information about its use and the options it supports, execute the

command with no arguments:

ZAPMID